BiosigNULL Changelog

Current as of: February 20th

V1.5:

Additions:

- Check_all_ports.py: Turns on each port 1 by 1 with a 10 second pause inbetween. This is used to evaluate which port corresponds to which coil. The coil is turned off (output 0, output off) when the 10 seconds are up.

- Optimize_single_sensor: Uses tuning curves to optimize voltages such that a given sensor has a given field (can be set to anything). Default bounds are 0-15V, will need to be changed within “solve_subset” function. Iterates over coils until tolerance is reached (uses as little coils as possible) using a greedy algorithm. Saves optimized voltages for each sensor, as well as residual fields.

- Set_all_voltages: Modification to previous script, removes feedback GUI and replaces with a single stop button, removing need to run kill_all_psu. Also includes a “read_in_voltages” tag which will allow selection of optimized voltages via excel sheet rather than manual entry. Shows the voltages and requires a Y/N confirmation before toggle. Voltage are rounded to two decimal places in selection.

Changes:

- Set_all_psu_voltages renamed to set_all_voltages_GUI

- Changed some output pathing in optimize_voltages

- Added a few comments to kill_all_psu

All edits by CL.

V2.0:

Additions:

- Added functionality for DPDT Relays.

- Initialize_arduino.py: .ino file that allows communication via Pyfirmata with Arduino MEGA. Run once on Mega. Copy of StandardFirmata.

- Flip_single_relay.py